Design And Calculation Report

Compression Spring

STRENGTH VALIDATION: Pass

INPUT DESIGN CONDITIONS

_________________________________________

DESIGN INPUT PARAMETERS

SPRING DIMENSIONS

(d) Wire Diameter: 1.60 mm

Active Coils: 4

(Do) Outside Diameter: 17.00 mm

(Lo) Unloaded Length: 32.00 mm

(Lp) Length at Preload: 28.00 mm

(Df) Required Deflection: 15.00 mm

Spring Ends: Closed and Ground

MATERIAL PROPERTIES USED FOR CALCULATIONS

Material: Stainless Steel Wire - Grade 3

Modulus of Rigidity: 71000 MPa

Density: 7800 Kg/m^3

CALCULATE RESULTS

_________________________________________

OUTPUT DESIGN PARAMETERS

Spring Index: 9.625

(Dm) Mean Diameter: 15.40 mm

(Di) Inside Diameter: 13.80 mm

(Lf) Length at Applied Load: 13.00 mm

(Fp) Preload: 15925.245 mN

(Ff) Applied Load: 75644.915 mN

STRENGTH VALIDATION

(P) Pitch of Unloaded Spring: 7.60 mm

Deflection at Preload: 4.00 mm

Total Deflection: 19.00 mm

Spring Constant: 3.981

Wahl's Stress Factor: 1.151

Shear Stress at Preload: 175471.557 kPa

Shear Stress at Applied Load: 833489.896 kPa

Tensile Strength: 1720000.000 kPa

Permissible Shear Stress: 860000.000 kPa

(Ls) Solid Length: 12.43 mm

Space between Active Coils: 6.00 mm